Condition report list

A condition report should include the following:

- a plan of the site including site dimensions

- site address

- aspect to north

- site levels or contours

- photos of site

- photos of any boundary pegs

- kerb-n-channel, condition and defects noted

- neighbours’ buildings and setbacks detailed

- neighbours' windows detailed

- fence location, condition and height

- site vegetation - trees and large shrubs

- footpath - condition and defects noted

- nature strips - note condition

- crossover location, condition and defects

- water, sewer and stormwater connection points

- gas and electricity connection points

- telecommunications connection point

- any pits are clearly illustrated on site plan

- Dial Before You Dig information (phone 1100).

Note that depending on local conditions there may be other requirements for inclusion in a site report. You may wish to discuss this with your trainer.