Condition report sample

Site address:	32 Bricker Street

Client:	Ms P Rightyo

Council:	City of Mandalay

|  | Yes | No | Notes |
| --- | --- | --- | --- |
| Survey pegs |  |  |  |
| Check survey |  |  |  |
| Site clearing |  |  | Small trees and shrubs |
| Rock |  |  |  |
| Fences |  |  | Three sides only, paling fences, good condition |
| Adjoining buildings |  |  | Garage on west boundary |
| Other conditions: |  |  | Existing well on-site to be filled. Confirm with engineer's report. |
| Water |  |  | Same side |
| Side |  |  | Far, UG |
| Power |  |  | OH |
| Gas |  |  | Nature strip |
| Phone |  |  | Nature strip |
| Sewer |  |  | Rear south east corner |
| Stormwater |  |  | Rear south west corner |
| Septic |  |  | Not required |
| Road |  |  | Bitumen, all weather |

|  | Yes | No | Notes |
| --- | --- | --- | --- |
| Footpath |  |  | Front boundary, good condition |
| Crossover |  |  | Crack across width |
| Flat site |  |  | 4 m fall to north east corner |
| Temporary fencing |  |  | To front only, 20 m approx |
| Notes: | 1. No space. On-site for spoil stock pile. 2. Adjacent garage may require protection. 3. Advise deliveries to use Waddington Road approach. 4. On-site access may be difficult in wet weather. 5. Poor visibility along Bricker Street - very busy road. |  |  |