Materials list

The house to be framed is a 133 square metre brick veneer house on a concrete slab with a truss framed concrete tiled roof. A brick single garage is attached.

This list is for a frame to be cut and stood on-site, including the roof.

Framing material:

| F5 pine studs | 90 x 35 | 230/2400 |
| --- | --- | --- |
|  | 90 x 45 | 14/2400 |
| MGP10 pine PLs | 90 x 35 | 18/3600 |
|  |  | 6/3900 |
|  |  | 4/4200 |
|  |  | 30/5400 |
| Merch pine | 90 x 35 | 23/4800 |
| Masonite braceboard | 2440 x 915 x 6.5 | 8 sheets |
| Mild steel angle bracing |  | 10/3600 |
| MGP10 pine joists |  | 10/2400 |
|  |  | 16/3600 |
|  |  | 14/4200 |
|  |  | 12/4500 |
|  |  | 6/5200 |
| Beams – MGP10 | 120 x 35 | 3/2100 |
|  | 140 x 45 | 4/3000 |
|  | 190 x 45 | 1/3600 |
|  | 240 x 45 | 2/4800 |

Framing material continued:

| F17 KDHW | 190 x 45 | 5/3600 |
| --- | --- | --- |
|  |  | 4/3300 |
|  | 240 x 45 | 1/3000 |
| MGHP10 pine | 90 x 45 | 37/4800 |
|  |  | 38/4200 |
|  |  | 8/3300 |
|  |  | 4/2700 |
|  |  | 12/2700 |
| Roof material |  | 14/3600 |
|  |  | 4/5400 |
|  |  | 2/6000 |
| F7 pine | 140 x 35 | 10/3600 |
|  |  | 4/4500 |
|  |  | 3/5400 |
|  |  | 2/6000 |
| LOSP fascia | 190 x 30 | 2/4500 |
|  |  | 3/540 |
|  |  | 2/6000 |
|  |  | 3/3300 |

Fixing material:

| Architrave/skirtings single |  |  |
| --- | --- | --- |
| MDF raw | 67 x 18 | 19/2400 |
|  |  | 2/3300 |
|  |  | 13/3600 |
|  |  | 11/3900 |
|  |  | 40/4200 |
|  |  | 4/4800 |
|  |  | 3/5400 |
|  |  | 3/6000 |
| Internal doors four panel primed and pre-hung |  |  |
| Internal sliding door | 2040 x 870 | 1 of |
| 2 RH | 2040 x 720 | 2 of |
| 2 LH, 1 RH | 2040 x 770 | 3 of |
| 3 RH, 1 LH | 2040 x 820 | 4 of |
| 1 RH | 2040 x 520 | 1 of |
| Corinthian/door unit | 2040 x 820 | 1 of |
| Latch sets – Lockwood 530 |  | 7 of |
| Privacy sets |  | 3 of |

Lock up material:

| Hardiflex eave sheets | 2400 x 600 x 4.5 mm | 15 of |
| --- | --- | --- |
|  | 2400 x 450 x 4.5 mm | 6 of |
|  | 1800 x 900 x 4.5 mm | 3 of |
| Divisional – white |  | 5 of |
| Merch pine | 70 x 35 | 80 metres |

This is a real list as estimated, tendered for quotations and then ordered.