Safe Work Method Statement

| Job Safety Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name of employer: | Safe T First |  |  |  | Date: | 23/11/07 |  |  |
| Workplace address: | 543 Sample Drive, Smartvllle |  |  |  |  |  |  |  |
| Overall task: | Assemble and erect wall frames |  |  |  |  |  |  |  |
| Activity sequence |  | Potential hazards/risks | Control measures |  |  |  | Who is responsible? |  |
| Step 1. Sort material |  | Stand close to the side of a timber pack and cut ties. Throw timber not required out of the way all over the site to obtain required timber. – physical injury, falling timber pack, tripping | Stand out of the way of a timber pack when cutting ties. Sort and stack material into lengths convenient to the job in a safe manner so the stacks do not fall onto self and others. |  |  |  | Builder/supervisor |  |
| Step 2. Pre-cut material |  | Pre-cut all timber components with a dropsaw. Throw cut components towards the job and do not stack them to save time. – physical injury, tripping | Set up a dropsaw properly, pre-cut all wall frame components and stack in an order convenient to the work area, but not so close to cause tripping and falling. |  |  |  | Builder/supervisor |  |
| Step 3. Assemble wall frames |  | Nail wall frames together with hammer/nailgun. Use nailguns in any direction. – physical injury | When using nailguns to assemble wall frames, be aware of other workers and do not nail in their direction to avoid injury from a flying gun nail. |  |  |  | Builder/supervisor |  |

| Activity sequence |  | Potential hazards/risks | Control measures | Who is responsible? |
| --- | --- | --- | --- | --- |
| Step 4. Stand up wall frames |  | Do not protect your body. – physical injury | Wear appropriate PPE, eg earmuffs, protective glasses, strong clothing, steel-capped boots, sunblock. | Builder/supervisor |
| Step 5. Make erected wall frames safe |  | Sit wall frames in approximate position, fix one end and leave the other end loose. | Nail bottom plate into place and temporarily prop all walls both ends to prevent them from falling onto self or other workers. | Builder/supervisor |
|  |  | Walk along the top wall plate to save time. – physical injury, falling, falling walls | Do not walk along the top wall plate. |  |
| Step 6. Housekeeping |  | Kick waste/off-cut material out of the way. | Keep all material stacked and remove all waste/off-cut material from the workplace. | Builder/supervisor |
|  |  | Run power leads/air hoses anywhere as long as they reach the job. – physical injury, tripping | Keep power leads/air hoses and tools away from where you need to walk. |  |

| Sub-contractor signature: |  | Builder/supervisor signature: |  |
| --- | --- | --- | --- |