Safe Work Method Statement

| Job Safety Analysis worksheet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name of employer: |  |  |  |  |  | Date: |  |  |  |
| Workplace address: |  |  |  |  |  |  |  |  |  |
| Overall task: |  |  | Working at heights |  |  |  |  |  |  |
| Activity | Potential hazards/risks |  |  | Control measures |  |  |  | Who is responsible? |  |
| Training | Untrained workers may not be aware of hazards that could affect themselves or others. |  |  | - Provide company induction, site induction and task training and certification as required. |  |  |  | Principal and contractor working at heights |  |
| Temporary access to heights | A fall from a ladder or other temporary access may injure workers. |  |  | - An industrial grade ladder should be used up to 6 m. - Secure the ladder safely at a 4:1 pitch. - The ladder must extend 900 mm+ past any safe step-off point. - An access tower should be used by heights above 6 m. |  |  |  | Contractor working at heights |  |
| Working at heights over 2 m | A fall or slide from a work platform may injure workers. |  |  | - Work from a safe work platform. - Provide fall protection over heights of 2 m. - Provide roof guard rails for roof edges if working on roofs. - All scaffolds and working platforms must comply with AS/NZS 4576 or AS 6001. - Protective non-slip footwear should be worn. |  |  |  | Contractor working at heights |  |

| Activity | Potential hazards/risks | Control measures | Who is responsible? |
| --- | --- | --- | --- |
| Working from ladders | A fall from a ladder may injure workers. | - Only insulated ladders are to be used for electrical work. - Only light duty work can be done where one hand is required to carry out work. - Workers are not to stand any higher than the third top rung. - Industrial grade ladders should be used up to 6 m. - Secure the ladder safely at a 4:1 pitch. | Contractor working at heights |
| Other workers working below or near workers who are working at heights | An object could fall and injure other workers below you. | - Prevent workers from working below other workers, where falling objects are possible. - Provide kick boards to prevent objects falling. | Contractor working at heights |
| Working with electric power tools | Workers could be electrocuted. | - All electrical equipment should be checked and tagged every three months. - Earth leakage protection should be provided. - Multi-plug outlet devices should be up to industrial standards and must comply with AS 3105. - Electrical leads should be clear of the ground and water. - No green sheathed leads are to be used. - Temporary switchboards are to be set up correctly. | Contractor working at heights |

| Activity | Potential hazards/risks | Control measures | Who is responsible? |
| --- | --- | --- | --- |
| Working near overhead powerlines | If a worker, plant or material come into contact with powerlines, the worker could be electrocuted. If scaffolding is within 5 m vertically and 4.6 m horizontally from powerlines on poles or plant within 3 m of powerlines on poles then additional precautions need to be taken. | - Identify where the powerlines are and provide warning signs if work is required in the 'No go zone'. - Isolate the power if possible. If this is not possible: - fit warning tiger tails to the powerlines - obtain written permission to work in the area from the electricity supply authority - complete a risk assessment and hold a site meeting - provide a qualified spotter. | Contractor working at heights |