Demolition daily checks

The following must be checked daily.

Make sure that:

- all openings and elevated free edges are properly guarded

- temporary hoardings, bracing, shoring and propping is tight and secure and in place as required to protect both workers and the public

- all potential fire and safety hazards are eliminated

- disconnected services are secured

- all hazardous materials are removed or covered as required by OHS legislation

- partly demolished elements are stabilised and loose materials are secured against bad weather

- emergency access routes are clear of debris

- all boundaries are secured against unlawful entry

- footpaths and public access are clear of demolished materials

- any hazards are properly lit, guarded and marked

- demolished materials have not been left on any floor or structure if their weight exceeds the safe carrying capacity of the floor or structure

- no structure or part of a structure is left in such a condition that it may become dangerous, such as collapse due to wind or vibration.