Soil report

Geebung Foundation and Building Consultants

Soil report part 1

212 Hilly Crescent

GEEBUNG   VIC   9876

Our reference:	9999

Report date:	14 October 2007

Site address:	Lot 404 Tally Avenue
	Beachside

Client:	Sandfly Drafting Services

Commission and building description

We have been asked to provide a site classification and footing recommendations for a brick veneer residence.

Site description

The proposed construction site is on the south side of the street and is virtually flat. Site drainage is fair. We anticipate that a deep site cut will not be required. Some small trees were noted near the proposed building at the time of our investigation (refer to conclusions and recommendations). The natural soil types are mainly sands and sandy clays developed on tertiary sediments.

Testing programme and findings

Two test sites were established and excavated to the depths indicated in the drill logs. Disturbed samples were collected and hand classified. The soil profiles encountered are shown in our log sections together with the results of in situ and laboratory testing (if applicable). No permanent water table was found during our testing programme. Any water seepage or 'perched water table' noted is shown in the log sheet.

Comments

The depth of the recommended foundation material may vary outside the test locations or if the site is excavated or filled in any way after the date of our testing. Some allowance should be made for the removal of organic matter, roots etc which may be found in small localised areas in the footing trenches.

| Test site | Trench depths (mm) | Recommended foundation material | Allowable bearing capacity (KPa) |
| --- | --- | --- | --- |
| 1 | 500 | Dense sand | 50+ |
| 2 | 600 | Dense sand | 50+ |

Site classification

We have examined soil profiles on this site and have classified these profiles as Class M.

Geebung Foundation and Building Consultants

Soil report part 2